# Exercise 4 Documentation

## Summary of Task:

Exercise 4 should take a string from the terminal emulator, such as putty, which is connected to the SCI1 input port. This string should be stored in a memory location. After the string is stored it should be manipulated such that it is either converted such that all letters following a space are uppercase (and the remaining characters become lowercase), or b. converted so that all letters are uppercase uses a switch in port p. A copy of The numerical characters will be stored in a new location. The manipulated string is sent to serial with a delay of 1 second. After this has been executed the numerical characters of the string will be sent to the 7 seg which will scrll through these

## Program Summary:

Note detailed documentation for each of these steps can be found in each exercises own documentation*

- Sets Baud rate and control registers for SCI

- Jump to receive subroutine for serial input

  - Wait until TDRE bit is set

  - Then transmit data into memory location

  - Once the CR is hit, add newline and CR

- Jump to Delay subroutine

  - Delay for 1 second

- Load the start of the receive string into x

  - Store an initial flag into memory locations, this is used to capitalise the first letter of the string if required

- Jump to the string manipulation subroutine

  - Check if the string should terminate

  - Check if char is alpha

    - Alpha chars get processed

    - Non alpha chars get skipped

  - Processeded chars have a function applied to them

    - All upper

    - All lower

    - Upper after a space

    - Upper for full stops

  - Non valid chars are skipped

  - Before moving to the next char the current char is tested if it is a space or fullstop, this information is stored in memory as a flag.

  - Position is incremented and branches back to start of the subroutine

- Jump to transmit subroutine

  - Wait until TDRF is set then transmit characters

- Return to the start of the loop

## Discussion Questions:

How can the 7-seg string be displayed if the microprocessor is currently waiting for information from the serial port?

What design and planning strategies did you use to incorporate all of the previous exercises into this task?

- For each module there is specified inputs and outputs which would be needed in the pipeline. For example the string manipulation tasks and the seven seg scrolling functions required a string which a way of determining the end of the string.

  - We tried to standardise aspects of the code such as what would be the terminating character

- We broke the integration task into a flow diagram of what should happen, and aimed at decoupling each of the exercises. Whilst this may have not been the most efficient approach, it makes testing much easier as for each stage of the pipline there are clear inputs and clear desired outputs.

  - This approach is best seen where exercise 1 the string is restored in the memory location and then for exercise 2 this new string is copied to a new memory location for the numerical characters, rather than attempt to do it all in one go.

  - This makes testing easier

- Other strategies also included reloading registers to ensure that previous operations would not affect the aims, this was particuly important as new sections of the code were appended or added to the old code.

How did you test the code?

## Diagram:

## Testing:

- Add in testing for